Supplementary Material

Prevalence of antibiotic resistance gene in different wastewater treatment systems and effluent-irrigated soils through metagenomic analysis

Hua Fanga,#, Miao Pub,d,#, Ailier jianga, Fuer haitia, Yubei Liua, Nuerla Ailijiangb,c,*, Anwar Mamate, Xifeng Tub,c,

aXinjiang Communications Investment Construction Management Co., Ltd

bKey Laboratory of Oasis Ecology of Education Ministry, College of Ecology and Environment, Xinjiang University, Urumqi 830017, P.R. China

cXinjiang Jinghe Observation and Research Station of Temperate Desert Ecosystem, Ministry of Education, Urumqi 830017, P.R. China

dXinjiang Institute of Nonferrous Metals Co, Urumqi 830022, P.R. China

eSchool of Chemical Engineering and Technology, Xinjiang University, Urumqi, 830017, PR China

#These authors contributed to the work equally and should be regarded

as co-first authors

*Corresponding author:

Email address: aljnel@xju.edu.cn (Nuerla Ailijiang)

This supplement provides additional information for the main paper.

S1. HPLC-MS/MS analysis

The column temperature was 40°C, the flow rate was 0.4 mL/min, and the injection volume was 5 μL. The mobile phases were 5 mmol/L ammonium acetate (A) and acetonitrile (B). The gradient program was set as follows: 0-0.25 min, 90% A; 0.25-3 min, 10-90% A; 3-4 min, 10% A; 4-5 min, 10-90% A; total running time was 5 minutes. Instrument conditions: capillary voltage: 4 kV; drying gas temperature: 600°C; drying gas flow rate: 3 L/min; nebulizer pressure: 35 psi. Antibiotic concentrations were quantified using the external standard method, with 100 ng/L of standard added to each sample to calculate the recovery rate of target antibiotics.

Fig. S1  Venn diagram of ARGs subtypes

Table S1 Soil sample collection coordinates

| Sample | North latitude | East longitude |
| --- | --- | --- |
| RS1 | 43°55’38’’ | 87°36’5’’ |
| IS2 | 43°58’0’’ | 87°40’56’’ |
| IS3 | 43°58’31’’ | 87°41’43’’ |
| IS4 | 43°55’33’’ | 87°31’29’’ |
| NS5 | 43°46’0’’ | 87°36’45’’ |

Table S2 ANOVA test of ARGs in different seasons

| Sample | F | p |
| --- | --- | --- |
| WWTS-HD | 0.4077 | 0.8430 |
| WWTS-MD | 0.8198 | 0.5370 |

Table S3 ANOVA test of ARGs in soils

| Sample | F | p |
| --- | --- | --- |
| Soil | 0.6292 | 0.6424 |

Table S4 ANOVA test of microbial genus in different seasons

| Sample | F | p |
| --- | --- | --- |
| WWTS-HD | 1.0537 | 0.3861 |
| WWTS-MD | 1.6721 | 0.1408 |